Neutron transmission measurements of natFe at nELBE

A. Experiment description

| - Main Reference | A.R. Junghans et al., Annals of Nuclear Energy (2025) 111363 | [A] |
| --- | --- | --- |
| - Facility | nELBE time of flight facility |  |
| - Neutron production |  |  |
| Neutron production beam | Electron ( ELBE with SRF injector) |  |
| Nominal average beam energy | 40 MeV |  |
| Nominal average current | 2 µA (13 - 20 pC bunch charge) |  |
| Repetition rate (pulses per second) | 100.0 kHz |  |
| Pulse width | 5 ps |  |
| Primary neutron production target | Liquid Pb-target |  |
| Target nominal neutron production intensity | 1011 n/s at 1 mA |  |
| - Moderator |  |  |
| Primary neutron source position in moderator | No moderation of neutrons |  |
| Moderator material Moderator dimensions (internal) (thickness, height×width×depth,…) | No moderator. The source volume is a 11 mm * 11 mm Mo tube with liquid Pb flowing through |  |
| Density (moderator material) | No moderator |  |
| Temperature (K) | Liquid Lead temperature 420 C |  |
| Moderator-room decoupler (Cd, B, …) | Not available |  |
| - Other experimental details |  |  |
| Measurement type | Neutron transmission |  |
| Method (total energy, total absorption, …) | Time-of-flight |  |
| Flight path length (m) (moderator –detector) | Target-detector 868.34(30) cm |  |
| Flight path direction | Single flight path only |  |
| Neutron beam dimensions at sample position (mm × mm, diameter in mm, …) | Sample in open field in front of 2.5 m long collimator |  |
| Neutron beam profile | uniform beam profile,see R. Beyer et al., NIM A723(2013)151–162 | [B] |
| Overlap suppression (Filter material and thickness, chopper, …) | No beam overlap, detection threshold is ca. 10 keV, Stripline-Kicker used to suppress dark current from injector |  |
| Other fixed beam filters | 5 cm Pb filter between neutron production target and transmission sample |  |
| - Detector |  |  |
| Type | plastic scintillator |  |
| Material | EJ 228 |  |
| Surface Dimensions (mm × mm. diameter in mm, …) | 200 * 20 mm2 |  |
| Thickness (mm) | 5 mm |  |
| Distance from sample (m) | Sample-detector 768 cm |  |
| Detector(s) position relative to neutron beam | detector in beam 0 deg |  |
| Detector(s) solid angle | Neutron beam larger than detector cross section |  |
| - Sample |  |  |
| Type (metal, powder, liquid, crystal) | pure iron (ALLIEDPUREIRON, exceeds ARMCO grade 4) |  |
| Chemical composition | iron, natural isotopic comp. |  |
| Sample composition (at/b) | 0.16948(54); 0.4250(14); 0.7645(25) |  |
| Temperature | 22-25 °C |  |
| Sample mass (g) | 77.1563(10); 193.4559(10); 348.01(1) |  |
| Geometrical shape (cylinder, sphere, …) | Right cylinder |  |
| Surface dimension (mm × mm, diameter in mm, …) | diameter 25.00(2) mm |  |
| Nominal thickness (mm) | 20, 50, 90 mm |  |
| Containment description | No containment |  |
| Additional comment | Transmission was measured for three different thicknesses. ALLIEDPUREIRON has a purity of 99.9 % Contaminants (Mn,Cr, Ni,Cu,P,Si,S,C,N,V,Ti,Al) on the level of 0.03%(Mn) or less than 0.01% all others. Molar mass value used: 55.85 g/mol Experiment took place 16.11.-24.11.2020 and 10.02-22.02.2021 |  |
| - Data Reduction Procedure |  |  |
| Dead time correction | Dead time measured per event, time of flight dependent correction, see R. Beyer et al., NIM A723(2013)151–162 Live time factor 0.86 – 0.99 | [B] |
| Back ground subtraction | Constant background subtracted ca. 80% of which is beam off background |  |
| Flux determination (reference reaction, …) | Relative measurement no flux determination |  |
| Normalization | See Main ref. [A] |  |
| Detector efficiency | Cancels in relative measurement |  |
| Self-shielding | URR self shielding correction not applied |  |
| Time-of-flight binning | 20/40.96 = 0.488 ns bin size from TDC |  |
| - Response function | Gaussian response function, described in ref.[A] |  |
| Initial pulse | Initial pulse length negligible (5ps) |  |
| Target / moderator assembly | L = 0.55 cm (1 sigma) from detector thickness and beam spot size on neutron production target |  |
| Detector | t = 0.205 ns (1 sigma) time resolution measured from width of bremsstrahlung peak in ToF spectra |  |

B. Data format

| Column | Content | Unit | Comment |
| --- | --- | --- | --- |
| 1 | Index Number |  | Count of data points |
| 2 | t | ns | absolute time-of-flight |
| 3 | Eu | MeV | Upper limit of kinetic energy of tof bin |
| 4 | El | MeV | Lower limit of kinetic energy of tof bin |
| 5 | E | MeV | Kinetic energy of tof bin (arithmetic mean of Eu,El) |
| 6 | Transmission |  | Transmission per time of flight bin |
| 7 | Uncorrelated uncertainty |  | Uncertainty from counting statistics and background subtraction |
| 8 9 10 11 12 13 | Total cross section Uncorrelated uncertainty Time of flight spectrum (Target IN) Time of flight spectrum (Target OUT) Relative background level Relative background level | b b | Effective total cross section per time of flight bin Uncertainty from counting statistics and background subtraction Target IN summed over full experiment, rebinned to 20 TDC channels ( 0.488 ns) dead time corrected before background subtraction (arb. units) Target OUT summed over full experiment, rebinned to 20 TDC channels ( 0.488 ns) dead time before background subtraction (arb. units) Target IN ratio of constant background to ToF spectrum Target OUT ratio of constant background to ToF spectrum |

RefErences

[A] A.R. Junghans, R. Beyer, R. Capote, D. Bemmerer, T. Hensel, T. Kögler, S.E. Müller, K. Römer, K. Schmidt, R. Schwengner, S. Turkat, J. Turko, S. Urlaß, Annals of Nuclear Energy 218, (2025) 111363

https://doi.org/10.1016/j.anucene.2025.111363

[B] R. Beyer, E. Birgersson, Z. Elekes, A. Ferrari, E. Grosse, R. Hannaske, A.R. Junghans, T. Kögler, R. Massarczyk, A. Matić, R. Nolte, R. Schwengner, A. Wagner, NIM A723 (2013) 151–162

http://dx.doi.org/10.1016/j.nima.2013.05.010

___________________________________________________________________________

___________________________________________________________________________